## Conditions:

Angular acceleration loads. When i simulated with this i saw nothing of concern so i blasted it to 1000 lol

Base force = 1250N → with factory of safety = 4.0

Force = 5000N

N cycles = 292000 → with safety factor = 2.0 → 60,000

## Funnel stuff

2.1557 in from the top, funnel diameter 3in

Simulation:

- Fixed at back end.

- Force applied to face of funnel and face of tube.

- Force applied: 1250N

- Max URES displacement: 1.675e-02 mm → 0.0659449 in

  - Located at edge of funnel

- Material: 1060 Aluminum alloy. Note: Funnel is tin, which is slightly weaker. Displacement might be higher. Reinforcement will probably be needed to survive fatigue and 29200 cycles lol

## Lever Lock

### Axial

Material for both: 1060 Alloy

Tested with a width of 0.35 inches → 0.44 pounds.

Thickness possibly could be 0.5 inches → 0.63 pounds.

For motor, test for max with 1 pound probably.

Sim specs:

- Fixed on both sides

- Force applied: 1250N normal to main face

  - *Force of coupling. This part should never see this much force. Wear and tear due to oscillation is the main load.

- Maximum deflection occurs in center: 2.656e-02 mm → 0.001045 in

- Fixed on one side

- Force applied: 1250N normal to main face

- Maximum deflection occurs in center: 1.880 mm → 0.074 in

  - Bottom right edge

Force=2500N, thickness = 0.5in

Force=2500N, thickness = 0.4in

### Centrifugal

speed: 1000 rpm

Speed: 1000 rpm

### Fatigue Axial

N=29200, F=2500N, T=0.4in

### Fatigue Centrifugal

Lever lock still requires some more design attention. I tried to design one that would give the motor a hard time so we could have some safety factor.

## Male (lol)

Static 1: Force: 2500N

- Suppressed U-joint connect (interference caused mesh issues)

- Fixed on back

- Centrifugal force: Angular velocity = 6.28 rad/s (yaw)

- Maximum deflection occurs in center: 7.568e-08 in

  - end

- Suppressed U-joint connect (interference caused mesh issues)

- Fixed on back

- Centrifugal force: Angular velocity = 4.19 rad/s (pitch)

- Maximum deflection occurs in center: 4.454e0-8 in

  - End

- Suppressed U-joint connect (interference caused mesh issues)

- Fixed on back & indent

- Centrifugal force: Angular velocity = 6.28 rad/s (yaw)

- Maximum deflection occurs in center: 8.663e-09

  - end

Speed: 1000 rad/s

Speed: 1000 rad/s

N=29,200 with Static 1. As you can see, this thing is chilling.

## Structural

Centrifugal

Due to turning. Assumed 90 degree turn in ¼ second → ω=25.12rads

Don’t know about angular acceleration yet.

Will have to remate male connections. Interference is preventing mating and simulations will not run.

Tension

Should not have to consider tension. Assumption is that control systems between vehicles are synchronous and do not create any velocity differential.

Oscillation

Shit

How do we do this

Fatigue

Top bar is 1060, bottom bar is 2024